## Baseline High down force velocity sweeps

Driver left side POV (Sai Patel)

- Midwing second vibration starts at 60 km/h

- Midwing second vibration violent at 93 km/h

- Rear wing side to side movement at 83 km/h

- Fix tape on front wing

- Front left wing hits ground at 98 kh/h but the front right side doesn’t (uneven suspension preload – doesn’t get fixed)

- Rear edge of midwing footplate vibration at 99 km/h

- Front wing right side amost bottoms out at 115 km/h

- Everything vibrating at 115 km/h

- Front no longer bottoms when stepping down to 70 km/h

Baseline High Down Force Yaw sweep

Driver Left POV (Sai Patel)

15 degrees

- Large vibration while turning into a slight vibration for midwing second

20 degrees

- Slightly larger vibration compared to 15 after settling